Electrical Network’s Modeling & Simulation Tools:

The State of the Art.

(Case Study of NMSS platform).

TAMALI Mohammed*1, ALLALI Malika2, BOUZIDI Boumediene3, & TOUMI Toufik4

ENERGARID Lab./Bechar University, URER/MS Adrar, Algeria, SONELGAZ Bechar, Algeria

Po Box 417, Kenadsa St., Bechar, Algeria

*1mtamali@gmail.com; 2malikaa450@gmail.com;3boudi08@yahoo.fr; 4toufiktoumi@yahoo.fr

Abstract: Scientific Software are classified into three categories,

business-specific tools designed by company itself and for its

proper needs and uses, industrial products planned and

designed for general/commercial uses and finally those

institutional designed by academic institutions depending on

research need of their laboratories.

For the first category, platform should meet purely localized

and specific tasks of company. Generally used by company

itself and named proprietary solution. Second category, tools

are planned and manufactured to meet a lot of tasks. These

platforms are a licensed solution. Third case is an institutional

category where specific codes are written first to meet a

specific need in a very specialized problem and then

supercharged with additional modules in next versions and

updates until a complete software would be born. These are

called institution-made solution.

Our paper discusses in a general overview about structural

and functional characteristics of commonly used software for

modelling & simulation of electrical networks.

Solutions from the three categories are studied and their main

features are described and compared according to criteria of

performance evaluation.

We introduce later, our NMSS solution (Network Modelling &

Simulation System). The modules and their functionalities are

exposed to highlight similarities and disparities with other

solutions.

Keywords:  Modelling; Simulation;  Numerical Library; GUI

Software Component; Electrical Network

I. INTRODUCTION

The proposal of a specialized product may, in most

occasions, be more general and meets to a part of the

customer's request. Therefore, the client uses must comply

with the conditionings of the platform.

In case of an academic solution, it should be noted that

the need for a development team is much needed. Major

obstacle to initiate research, since no support and

stimulation are observed unless this represents a major topic

of tasks to be performed by research Laboratory.

Many software products for modelling and simulating

electrical networks already exist.

Products efficiency, used to design an eventual electrical

network is significantly dependent on modules set to predict

the behavior of studied system under normal or abnormal

operating.

Manual evaluation is possible for small electrical

systems but is expensive in engineering time. In addition,

the manual computation methods are not obvious for safety

reasons and under cases where the all design is much more

expensive and requires more efforts. The consequences of a

faulty design of electrical network model can induce:

• Danger threatening human lives if adopted.

• Increase of total costs of design and its

implementation.

• Poor Reliability/robustness of outcomes

• High costs for repair operations and maintainability.

• Causing damage to the network (incorrect decision

due to misalignment of the load level with respect to

supply request).

For designing reliable electrical networks, engineer

needs specialized potential human resources and appropriate

set of tools.

Modern numerical methods provide a fast and accurate

calculation of electrical system  behaviours and network

topologies, once data and parameters were given to platform

running required software. Study of electrical network can

be quite large and it is very desirable that the engineer has a

reliable software tool that uses GUI to acquire data. In the

same way, engineer must have access to calculation results

in a clear, and unambiguous report.

In this paper, we examine some-well known platforms

software issued from academic and/or commercial category

and designed by professionals of electrical engineering field

and their development team. These teams have acquired

practice over the time. Software products are presented in

what follows without any classification or preference

ordering but just an enumeration as an example.

Notifications of characteristics are even reported.

NMSS, a solution we developed is exposed and

compared according to the same criteria.

II. EVALUATION CRITERIA MODEL

Software tools studied are technically compared under

the following criteria and recommendations:

- The Graphical User Interface, GUI

- Accessibility to visual components

- Modularity of all platform

Journal of Electrical and Control Engineering                                                                                                                         JECE

JECE Vol. 3 No. 5, 2013 PP. 1-12 www.joece.org/ © American V-King Scientific Publishing

2

- Presence of standard data libraries

- Possibilities for comprehensive reports

- The portability of the tool

- Interoperability

- The ability to exchange data.

To understand, scientific software, we need to see from a

specific point view where structure is exposed in the

following illustration given at Fig. 1:

Fig. 1. The main structure of scientific software.

In general, a scientific application must respond

effectively to requests emitted by user who needs to

manipulate the object model under study. These objects are

represented by two models, in one hand a description in data

format to reprocessing and in other a visual format to

display and manipulate on a screen.

The application can organize the requests according to

synchronous and asynchronous modes. Queries are issued

and then sent to the inference engine to be processed after

which results emerge and then re-emitted to user and/or

saved in database under a new knowledge and rules format

(Fig. 1).

In the following sections different products will be

displayed and then a technical description is given.

Available characteristics are given and thus discussed.

The qualifying order of exposed solutions is efficiency

or not against of academic use,  to manipulate electrical

network models on a computer screen.

III. OVERVIEW ON PROPRIETARY SOLUTIONS

A. Case of Powerworld Simulator (From Powerworld Inc):

PowerWorld Simulator (Fig. 2) presents a variety of

modules that can be used in standalone mode (such as OPF,

ATC, PVQV, SCORF, SimAuto ...).

Fig. 2. Main window of PowerWorld Simulator 17 (from PowerWorld Inc.

www.powerworld.com).

The software is provided with online documentation

which can be used before beginning the study. This software

is highly interactive. This feature has many advantages but

also some disadvantages are shown in handling. Include:

• The lack of memory operations and parameter changes

performed successively hence the impossibility to find, after

certain tries, the initial situation. It is strongly recommended

to make a backup of the initial situation and saving

intermediate situations that you want to reuse later. It is

possible to associate a single line diagram Pwd file in many

cases ‘.Pwb’ files1.

• The activation of the majority of controls (AGC limit

active power generators ...) should be performed at three

levels:

1. At the global level for the entire network via an

option setting for general simulation;

2. In each region through a parameter by activating the

dialog box on each region;

3. At the equipment itself (eg generator) through a

parameter by activate the dialog of this equipment.

Note that these settings are also available in the

summary tables by typing of equipment ("information box"

tables).

- This three-level activation is also present in the

specification of constraints Optimal Power Flow.

- It is recommended to perform the operation "Reset

to flat start" menu "Simulation”. Indeed, this

operation, which is intended to reset the flat voltage

profile "1+j*0" and could, also resets to 0 the

production of reactive power of all generators

including PQ type generators. Reactive instructions

of these generators are therefore lost.

- Specifying display filters to limit the display of

various sizes in regions for which this filter is

enabled.

The majority of options and general settings necessary

for carrying out the work are already set appropriately in the

situation provided. To enable an understanding of the use of

the software, it is nevertheless advisable to check the

choices and values adopted.

Actual versions have been maintained and corrected to

meet the standards requested. The PowerWorld is the most

used software in academic institutions built to meet

university need of visual simulations of electrical networks

tool.

B. Case of SPARD Simulator® from Energy Computer

Systems 2012

SPARD® software suite mp Power (Fig. 3) is an

application environment for computing and simulation of

electrical networks. Each module can operate independently

of the graphics module GUI which is a strong quality. This

environment consists of six basic modules:

- Drawing network module

1 note the difference between pwb and pwd

Journal of Electrical and Control Engineering                                                                                                                         JECE

JECE Vol. 3 No. 5, 2013 PP. 1-12 www.joece.org/ © American V-King Scientific Publishing

3

- Data entry module

- Simulations based module

- Coordination and Protection module

- Additional applications (Engine Start, Optimal

reconfiguration, placement of sub-stations, Service

Restoration, and probabilistic fault simulation, etc...)

- Reports editing module.

The individual modules are simultaneously available to

the end-user. If the network system state is, for example, in

its steady state, so that a transmission line can be added and

configured graphically with new data. Graphical area model

is immediately updated accordingly to changes made.

Without any experience, the end-user can easily handle and

manipulate GUI2 with ease, drawing nodes, transmission

lines, transformers ...etc. by a simple drag & drop operation

from the toolbar. Any changes occurred thereafter is run,

with the same flexibility, and reflected by equivalent

drawing updates.

SPARD Power system is "intelligent". If the screen data

option is displayed, just select an object (node, relays ...) on

the graph related and then appear properties automatically.

The same data can be imported from module SPARD®

Distribution and even from the Microsoft Excel spreadsheet.

Simulation of scenarios is a matter of few clicks.

SPARD has three levels of network conditions, the steady

state, the transient and the transient effect under protection.

Fig. 3. Main window of SPARD®Power 2012

(http://energyco.com/english/soluciones/spard-power/)

C. Case of ETAP from Operation Technology

The basic module of ETAP Base (Electrical Transients

Analyzer Program)  (Fig. 4) is a package with standard

functions and capabilities included in all distributions sold.

The basic module includes functions such digraphs single

line editor function, which is one with the most powerful

force making the core controller of all  practical cases

studied. It is designed to allow engineers to quickly build

and manipulate models and try instantiated through its user

friendly  GUI with a variety of computational tools and

manipulation.

2 Graphical User Interface

The base module includes an extensive library of

technical data and parameters ready for use and ISO

compatible. It is designed around an open and organized

core. ETAP Wizard, one of other base components, is

designed to allow the automation of many tasks, scenarios

and reports creation.

Fig. 4. Windows main spreadsheet software ETAP (www.etap.com).

The main features are:

- Unlimited number of nodes (license).

- Supports variety of devices (organized in libraries).

- Various modes of one-line diagram representations.

- Unlimited configurations and scenarios.

- Extended manipulation of technical properties.

- Support cables sensitivity against temperature or

frequency.

- Unlimited nesting subsystems in a one model.

- GUI designed on rules of art.

- Import OLE objects (eg. Microsoft Office).

- Automated typical data input.

- AC-DC Systems integration.

- Grounded Integration into single-line diagram.

- Library controlled and maintained by user.

- Reports and planning tasks editing.

- Enhanced accessibility.

- Normalized cables (NEC, ICEA).

- Smooth configuration and edition of cables.

- Assistant to the management of scenarios.

D. Case of EasyPower-Power Flow Analysis from ESA Inc

Fig. 5. main window of EasyPower tool (www.easypower.com).

Journal of Electrical and Control Engineering                                                                                                                         JECE

JECE Vol. 3 No. 5, 2013 PP. 1-12 www.joece.org/ © American V-King Scientific Publishing

4

EasyPower (Fig. 5) is a graphical interface dedicated to

the development and study of  electrical network drawings

(real or during simulation). It allows,  through,  its various

graphical components to compose brick by brick electrical

network to be  examined. Its great feature is possibility to

study and to simulate an arc-flash event occurring during an

open media transmission or distribution, which can be

dangerous if executed many times on the ground by agents

less attentive.

EasyPower combines between a lot of modules

SmartDesign,  SmartPCD,  ArcFlash, and IEC short-circuit,

EasyPower@ PowerFlow, EasySolv, safetyTracker.

During an end-user session, all the capabilities of the

platform are accessible through few mouse clicks. It is also

important for designers on EasyPower to unify the access

point for all manipulations. EasyPower calls this ability

'Focus'. There are five types of 'focus':

- Editing the controls and equipments database

- Analysis of shorts-circuits

- Analysis of load transit

- Harmonic analysis.

- Coordination analysis equipment.

Each 'focus' is represented by its own menu and toolbar,

showing only those controls attached to it, which gives a

greater clarity to the network editing.

E. Case of CYME OPF from CYME

Fig. 6. CYME OPF main Window (www.cyme.com)

PSAF (Fig. 6) is a suite of software tools with the major

features to analyze energy networks and allows the

simulation of equivalent models.

A wide range of network types and their related

equipments and controllers are possible to be used under

CYME-  PSAF and are integrated in libraries according to

industry standards.

Each device provides various type and opportunities for

being chosen in conducted simulations.

PSAF provides two types of interfaces, graphical one

and tabular allowing other mode of accessibility. The first

one for drawing line diagram of the resource, the second for

editing reports, computing and accessing results, printing or

for adaptation of simulations to scenarios.

Using CYME-  PSAF adds significant advantages to

perform project planning and problem solving.

PSAF is directly used in three-phase mode of utility or

industrial versions.

F. Case of ERACS Software Information from ERA

TECHNOLOGY LTD

ERACS software (Fig. 7) it, always, been at the

forefront of engineering tools development. It always takes

into account the continuous pressure for easier and

operational software to meet the growing needs of technical

modern engineering. For many years now, ERACS name

has been synonymous with quality, precision, accuracy;

ERA Tech has aligned its adaptability, with changing market

needs.

Alignment to good practice has always been the criterion

for a successful design and is a major advantage for ERACS

end-users in time and efforts to save for manipulating

network analysis.

As expected, in PC software are more fully integrated.

The data is entered only once in a central database which

makes their management a very simple procedure.

A wide range of facilities are been incorporated to allow

users to simulate the electrical network, the results of such

treatment is to give end-users ability to quickly and easily

assess their accuracy. Networks can be simulated in ERACS

in two types, radial or completely interconnected but a

mixture of both is also available.

Fig. 7. Main application window ERACS (www.eracs.co.uk).

In general, ERACS combine between a set of

possibilities. ERACS with GUI facilitates access to

databases and more:

- The calculation of Load-Flow in steady state mode.

- Faults calculating to simulate defects application

such as short-circuit.

- IEC909 default which simulates the application of

fault conditions using techniques IEC 909.

- Allows harmonic injection and processing of their

proliferation.

- The harmonic impedance as replacing the network

impedance against many harmonics and identifies

the resonance conditions.

Journal of Electrical and Control Engineering                                                                                                                         JECE

JECE Vol. 3 No. 5, 2013 PP. 1-12 www.joece.org/ © American V-King Scientific Publishing

5

- Transient stability for the dynamic study of

perturbations applied to the network.

- Protection and coordination analyzes and the

placement of protective devices and their interfacing

under fault conditions.

- Universal Dynamic Modeling (UDM) used to create

the model controller (AVRs and governors) built

around a range of construction sets which are

validated and easily understood by engineers.

- 150 sets of busbars in standard mode and 1,500

busbars for the ERACS extended version are

available.

- A network-distributed version for collaborative

tasks is also available.

- A rich library of electric components data.

G. Case of DigSILENT from PowerFactory.

Fig. 8. Main Window DigSILENT (http://www.digsilent.de/)

PowerFactory DIgSILENT software (Fig. 8) is a tool to

aid the engineer to analyze electrical networks. It was

designed as an integrated package and with advanced

interactive tool devoted to the treatment of electrical

networks, the analysis control to accomplish the main goals

and objectives of organizing and optimizing tasks related to

electrical power systems.

DIgSILENT for its fourteenth (Fig. 8) version was the

first software of network analysis in the world with

integrated graphics. The diagram is introduced in an

interactive way, by editing the static or dynamic settings.

The PowerFactory package was designed and developed

by skilled engineers and programmers with many years of

experience in analysis of electrical networks. The accuracy

and validity of the results obtained with this package have

been confirmed in many cases implemented by

organizations involved in the control electrical networks

operations.

To meet the requirements of the analysis of current

network designers, DIgSILENT offer through all available

features, a collection of many different software modules.

The development team wants its product offering best usage

trough the system's network configuration with interactive

access or connected to the SCADA3 system.

Using a database with all information required in

relation with any material equipping standard networks (eg

data line, data generators, data protection, data plans

harmonics, data controller). We can easily run DIgSILENT

PowerFactory, all in the same simulation environment.

Some of these functions are current discharge treatments,

calculation of short-circuit analysis with the presence of

harmonics, protection coordination, stability calculation and

modal analysis.

H. Case of NEPLAN from Bussarello+Cott+Partner Inc.,

Erlenbach Switzerland

Product NEPLAN (Fig. 9) is a software suite that

integrates the following modules:

- Base module for graphic edition.

- Transit charges calculation.

- Transit under dynamic loads.

- Transit loads with load profiles.

- Distribution of powers.

- Load distribution Problem.

- Short-circuit Plan.

- Protection Calculation.

- Harmonics Plan.

- Start engine.

Fig 9. Main Screen of NEPLAN tool (http://www.neplan.ch).

- Network State Snapshots

- Analysis in dynamic mode

- Network stability

- Analysis of Investment

- Analysis of network reliability

- GIS interface

- ODBC database interface and connectivity

With NEPLAN, networks with any size can be designed

and subsequently treated without limit.

The elements taken into account in any design are:

- Nodes

3 For Supervisory Control And Data Acquisi